----------------------------------

Setting the COM port for EPHTUNE |

----------------------------------

The default COM port for EPHTUNE is COM 1.  The COM port can be changed

to COM 2 in either of two ways.  The COM port can be specified on the command line,

e.g.,

EPHTUNE 2 <CR>

or, the GFLG environment variable can set to indicate the COM

port.

SET GFLG=-c2

Note that the 'c' in -c2 must be lower case and only ports 1 and 2 are

valid.

---------------

Using EPHTUNE |

---------------

EPHTUNE allows the adjustment of 3 of the EPH Series radios operating

parameters: receiver tuning slope, transmitter high power set, and

transmitter low power set.

*General*

Connect the radio to an IBM PC or compatible computer using the EPH

series programming/test cable.  This cable allows access to the antenna

and audio connectors while programming the radio.

*Key functions*

PgUp/PgDn	Increases or decreases the value of the highlighted

setting displayed on the screen.  DOES NOT alter this

value in the radio.

keypad +/-	Increases or decreases the value of the highlighted

setting and updates the highlighted value in the radio.

ENTER		Updates the radio with all values displayed on the screen.

F2		Updates all screen values with those obtained from the

radio.

numeric keys	Directly enters a value in the highlighted field.

ESC		Exits the EPHTUNE program.

*Receiver front end slope*

To adjust the receiver front end slope, set up the test equipment to

measure sensitivity.  Program the radio and RF source to a frequency

near the high end of the band.  Adjust the coils in the receiver for

maximum sensitivity.  Program the radio and RF source for a frequency near

the low end of the band.  Invoke EPHTUNE.  If the radio EEPROM is new or

has been erased, enter an initial receiver slope setting of 26 and press

<ENTER>.  Using the '+' and '-' keys on the keypad, adjust the receiver

slope for maximum sensitivity.  This setting can be any number between 0

and 255.

NOTE:  Because of RF interference from the computer, the radio may not

meet the sensitivity specification until disconnected and removed from the

immediate vicinity of the computer.

Transmitter power settings

Set up up the test equipment to measure transmitter power.  Program the

radio for the desired transmit frequency, and set the HI/LO power switch

as desired.  On the computer, use the cursor keys to move the cursor to

the HI or LO power setting.  Key the radio and measure the transmitter

power.  Unkey the radio.  Adjust the appropriate power setting and repeat

as necessary.  The transmitter power settings can be any number between

0 and 63.

NOTE:  	The radio does not commumicate with the computer while

transmitting!  Unkey the radio before changing any radio

settings.  An attempt to modify these values while trasmitting

will NOT damage the radio, but will cause an error to be displayed

at the bottom of the screen.  To clear this error, unkey the radio

and press the F2 key to re-read the radio settings.