Per Channel

The Per Channel menu option gives the user access to the features that are enabled/disabled on a Per Channel basis. Selection of the option opens the Per Channel window which contains the Per Channel features collected together in functional groups under the headings:

Common Options

Display

Transmit (TX)

Receive (RX)

Transmit Options

Receive Squelch

Single Tone Encode

Single Tone Decode

Common Options

_____________

Allow Channel Selection

Check  this check box to enable the channel for use.

With the A2, 4-channel radios, up to sixteen channels can be programmed into the radio's codeplug. Only the first four "allowed" channels can be accessed through the keypad. For Selectone Smartrunk II systems, ensure that all trunked channels occupy the channels starting from channel 1.  For these trunked channels, do not check the 'Allow Channel Selection'. Any "conventional", i.e. non-trunked, channels in the system should be above the trunked channels, and for these conventional channels, ensure that the 'Allow Channel Selection' box is checked (so up to 4 conventional channels are possible).

With the A3, 128-channel radios, channels that are not "Allowed" will not be seen in the channel list as the user scrolls through the channels.

Pre-emp/De-emp On

Voice transmit pre-emphasis and receive de-emphasis used in voice communications can be enabled per channel. It should be set on in all standard applications.

Talkaround Allowed

When Talkaround is selected, the transmit frequency on a channel is switched to be the same as the receive frequency on that channel. The feature is used to allow one radio to communicate directly with another without the need to use a repeater.

Checking the Talkaround Allowed check box enables the feature on that channel only. To use the feature, the user must have programmed an option button as Talkaround; the button is then used to select/deselect Talkaround as required. When Talkaround is selected, for A3 display radios, the Talkaround icon is activated ; for A2 non-display radios, the Green option LED is illuminated. If an attempt is made to select Talkaround on a channel with Talkaround disabled, the button error alert will be sounded.

Once selected, Talkaround remains active across channel changes provided that the channels have Talkaround enabled. When a channel is not enabled, Talkaround will not operate but will again operate when a channel with Talkaround is selected.

It is not possible to select Talkaround while the radio is actively scanning, but it is possible to start the radio scanning while Talkaround is selected.

Display

______

Display Alias

If this is enabled, the alias will be displayed  instead of having one of the pre-defined channel numbers displayed. This allows the user, for example, to number a channel according to an existing channel plan (001, 014, 016, 034 or C01, C02, C03). To enable the channel alias for a channel, check the identified check box.

Display Alias is mutually exclusive to Display Channel Number.

Display Channel Number

If this is enabled, the default channel number will be displayed on the radio display.

Display Channel Number is mutually exclusive to Display Alias.

Transmit (TX)

_________

Tx Frequency (MHz)

A designated frequency in MHz for transmitting messages and signals. Frequencies must be within the specified band for the radio and be divisible by 5 kHz or 6.25 kHz. The required frequency is keyed in to the identified entry field. Each channel must have a Tx frequency defined, if the selected frequency is 0 (zero), the channel will operate as receive only.

RF Power

RF Power is selected through the identified spin box. The available options are Power 1 and Power 2.

TX PL/DPL Mode

Private Line (PL or CTCSS) signalling consists of a sub-audible tone that can be sent out with voice or with tone signalling. When a radio is programmed to transmit with a PL code on a particular channel, then it will send the tone signal when keyed on that channel. When the radio is dekeyed, the tone signal is stopped, and a PL 'reverse burst' is generated. The reverse burst consists of PL encoded with a 240 degree phase shift. It is sent to indicate the end of transmission for the decode radio.

Digital Private Line (DPL) signalling is a digitally coded signal that can be sent out simultaneously with voice or with tone signalling when a radio is programmed to transmit with a DPL code on a particular channel. The radio will then send a digitally coded signal when keyed on that channel. When the radio is dekeyed, the digitally coded signal is stopped and the Turn Off Code (TOC) is sent to indicate the end of transmission for the decode radio.

Inverted DPL should not normally be used but will be retained for special applications such as a 'flipped' audio path or a multi-hop repeater. Inverted DPL Encode inverts all the bits in the DPL code before sending it. It should be noted that inversion is applied independently to DPL Encode and DPL Decode, i.e. there is no constraint that if inversion is applied to encode it must be applied to decode and vice versa.

Implementation of PL/DPL selection is achieved using one of two combo boxes separated by an entry field. The first combo box allows selection of standard PL codes. The first option in the box is Disabled, that is followed by all of the standard PL codes in the form 'XZ - 67 PL', and the last option, is User Defined PL. This allows for the selection of non-standard PL, the required frequency being keyed in to the entry field.

The second combo box is used for DPL code selection. The options available in the box are 'Disabled', and all standard DPL codes. If a DPL code is selected, the DPL Invert and DPL TOC selections must be addressed.

It should be noted, that PL and DPL may not be used on the same channel, i.e. PL transmit (encode) and DPL receive (decode) or vice versa.

DPL Invert

Select this check box only if your DPL selection requires inversion.

DPL TOC

If you have selected DPL encode, you will need to check the Turn Off Code check box to enable the feature to ensure that the transmission is correctly terminated.

Receive (RX)

_________

Frequency (MHz)

A designated frequency in MHz for receiving messages and signals. Frequencies must be within the specified band for the radio and be divisible by 5 kHz or 6.25 kHz. Each selected channel must have a receive frequency defined, 0 (zero) is not permitted.

RX PL/DPL Mode

Private Line (PL or CTCSS) signalling consists of a sub-audible tone that can be sent out with voice or with tone signalling. When a radio is programmed to transmit with a PL code on a particular channel, then it will send the tone signal when keyed on that channel. When the radio is dekeyed, the tone signal is stopped, and a PL 'reverse burst' is generated. The reverse burst consists of PL encoded with a 240 degree phase shift. It is sent to indicate the end of transmission for the decode radio.

Digital Private Line (DPL) Squelch is a digitally coded signal that can be sent out simultaneously with voice or with tone signalling when a radio is programmed to transmit with a DPL code on a particular channel. The radio will then send a digitally coded signal when keyed on that channel. When the radio is dekeyed, the digitally coded signal is stopped and the Turn Off Code (TOC) is sent to indicate the end of transmission for the decode radio.

Inverted DPL should not normally be used but will be retained for special applications such as a 'flipped' audio path or a multi-hop repeater. Inverted DPL Encode inverts all the bits in the DPL code before sending it. It should be noted that inversion is applied independently to DPL Encode and DPL Decode, i.e. there is no constraint that if inversion is applied to encode it must be applied to decode and vice versa.

Implementation of PL/DPL selection is achieved using one of two combo boxes separated by an entry field. The first combo box allows selection of standard PL codes. The first option in the box is Disabled, that is followed by all of the standard PL codes in the form 'XZ - 67 PL', and the last option, is User Defined PL. This allows for the selection of non-standard PL, the required frequency being keyed in to the entry field.

The second combo box is used for DPL code selection. The options available in the box are 'Disabled', and all standard DPL codes. If a DPL code is selected, the DPL Invert and selection must be addressed.

DPL Invert

Select this check box only if your DPL selection requires inversion.

Transmit Options

_____________

Time Out Timer Enabled

Checking this check box enables the Tx Time Out Timer for the channel. The parameters for this feature are set up in the Transmit group in the Per Radio window and are discussed under that heading in this User Manual.

Forced Monitor Enabled

Checking this check box enables the Forced Monitor feature for the channel. The parameters for this feature are set up in the Receive group in the Per Radio window and are discussed under that heading in this User Manual.

Transmit Allowed

The Transmit Allowed feature is used to prevent the user from being able to transmit on channels that are already being used. The criteria are checked when the user attempts to transmit by, for example, pressing the PTT.

The selectable Transmit Allowed criteria are:

Always No Transmit Allowed criteria; the user is always allowed to transmit.

If Channel Free The radio will inhibit transmission if carrier is currently being detected on the receive frequency.

PL/DPL Lockout When the channel is busy, the radio will inhibit transmission unless your selected PL/DPL is present on the receive channel, i.e.: your user group is currently active on the channel and your selected PL/DPL is being detected.

Past PL/DPL Lockout When the channel is busy, the radio will inhibit transmission unless your selected PL/DPL has been present on the receive channel, i.e. you and/or your user group are active on the channel and your selected PL/DPL has been detected, BUT there has been no loss of received carrier since the correct PL/DPL was detected.

Full Lockout

Standard or Past  PL/DPL lockout (Busy Channel Lockout) does not disable the mobile radio user's ability to monitor a channel. In those instances where even monitoring is not permitted, e.g.. a shared repeater, Full Lockout prevents any form of monitoring of the channel. To enable this facility on the selected channel, check the check box.

In Autoreset Period

When checked, this causes the Transmit Allowed criteria  to be applied during autoreset.

Receive Squelch

_____________

The radio supports programmable squelch settings for each of its receive, monitor 1 and monitor 2 squelch modes. Squelch settings determine what signalling the radio must detect if it is to unmute.

Receive Squelch Enables the user to select the appropriate level of standard receive squelch they require depending on their preference or system configuration. This allows the user to ensure that only the correct call/messages are received.

Monitor 1 and 2 Squelch This allows the user to pre-determine the squelch settings which will occur when the monitor button is pressed for either a short press (Monitor 1) or a long press (Monitor 2).

Squelch Options

The options available for selection as Receive, Monitor 1 and Monitor 2 are:

Carrier This option enables the radio to unmute when carrier is detected on the channel.

PL/DPL Squelch This option enables the radio to unmute while the PL/DPL frequency is detected on the channel.

Tone Squelch This option enables the radio to unmute on receiving a Single Tone selective call on the channel.

PL/DPL + Tone Squelch This option enables the radio to unmute if the correct PL and Tone Squelch are detected together on the channel.

Open The radio is permanently unmuted (also known as 'Volume Set' on other radios).

Autoreset

This feature determines the way in which the radio resets when Autoreset is enabled. The Autoreset Mode options are: Carrier Override, Carrier Independent, Manual Reset, Disabled.

Autoreset is a feature that is typically associated with tone squelch operation but is also applied to PL squelch. On receipt of a selective call or on dekeying, the radio will enter autoreset mode in which tone squelch is defeated, i.e. if the current squelch setting is 'tone squelch' then the radio will operate in carrier squelch, if the current squelch setting is 'tone and PL squelch' then the radio will operate in PL squelch. Additionally, it is possible to specify that PL squelch is defeated during the autoreset mode using PL Override which is discussed below.

The Autoreset timer allows the user a set period of time to respond to a call by just pressing the PTT switch and therefore not having to call the other party back each time they wish to speak to them. This reduces the need for user intervention during calls and also ensures that the radio squelch resets after a call has been completed, thereby preventing users overhearing the next call on the channel.

Carrier Override Detection of carrier within the auto reset period will reset the timer which will start again when the carrier is lost. Transmission by the radio will also reset the timer. If the channel has PL decode enabled then auto reset will count down whilst PL is not being detected and will be reset whilst PL is detected. Exit from monitor operation will auto reset the radio even if the auto reset timer has not expired. Hence, the user will be able cause auto reset by tapping the monitor button provided monitor operation is enabled.

Carrier Independent This differs from auto reset 'carrier override' in that the auto reset timer will not be affected by the presence of carrier or PL. On expiry of the timer the radio will auto reset.

Manual Reset The radio will remain in the auto reset mode until the monitor button is tapped. Manual reset will only be available if monitor is enabled.

PL Override

PL Override is used in conjunction with Autoreset. Check the check box if the user requires PL to be defeated during the operation of Autoreset.

Single Tone Encode

________________

Single tone encode may be initiated before speech transmission on PTT Key and after speech transmission on PTT Dekey. Single tone encode may also be initiated by option buttons that have been programmed for single tone encode (Call 1 and Call 2).

Tone on PTT Key

Select the required tone, J, K, L or M, or '-' (Disabled) in the combo box.

Mode

When required, the single tone keyup tone will be transmitted after the radio has been successfully keyed and before speech transmission starts. The requirement is determined by two mutually exclusive options, offered for selection in the combo box:

Once Only The single tone will be sent on depression of the PTT if the radio is either, not in autoreset mode or, is in autoreset mode but a call has not yet been started. This option ensures that a tone will be sent only when an attempt is made to initiate a call and not, during or in response to a call. This option also caters for the situation whereby autoreset mode is entered via the forced monitor feature.

Every PTT The single tone will be sent after the radio has keyed and before speech transmission starts.

Tone on PTT Dekey

The single tone dekey telegram will be sent out after speech transmission, just before the radio dekeys. Select the required tone, J, K, L or M, or '-' (Disabled) in the combo box.

Call 1 Tone

This combo box enables the selection of the tone to be transmitted when the option button programmed as CALL 1 is pressed. When this occurs, the programmed single tone, along with pretime and hold time, will be encoded. If the feature is not required on the current channel, - (Disabled) should be selected.

Note: The complete single tone, including hold time, will be transmitted even if the option button is released before the single tone transmission is complete.

Call 2 Tone

This combo box enables the selection of the tone to be transmitted when the Option Button programmed as CALL 2 is pressed. When this occurs, the programmed single tone, along with pretime and hold time, will be encoded. If the feature is not required on the current channel, '-' (Disabled) should be selected.

Note: The complete single tone, including hold time, will be transmitted even if the option button is released before the single tone transmission is complete.

Sidetones

Sidetones are an aspect of the Single Tone Encode feature. When the Sidetones check box is checked, the user is able to hear the side tones when a single tone is encoded; the side tones providing an audible indication that the tones are being transmitted. The side tones will consist of the actual tone(s) being sent rather than just a fixed frequency alert.

Single Tone Decode

________________

Decode Tone

For A3, 128 channel radios only. Typically, this feature defines the radio's support of selective calling via single tone decode. The Decode Tone combo box allows the user to select the required single tone from the range J, K, L and M, in support of selective calling, or '-' (Disabled), if selective calling is not supported on the current channel.